Squarebird front foam and upholstery

The following guidelines are based upon my personal experience in fabricating foams and doing Squarebird front foam and upholstery jobs;

Steps to follow when Hog Ringing the Upholstery over the Foams and onto the Frames

- Center foam on frame.

- Contact glue canvas cloth sleeves to the perimeter of frame.

- Insert wire splines into foam slice cut (coat hangers make good spline material).

- Hog ring the splines to frame thru back fabric of seat foam thus anchoring the foam to the frame.

- Insert wire splines into the pocket inserts of the upholstery

- Place the upholstery over the foam and insert the upholstery attached spline sleeves into the foam slice cuts.

- Hog ring the inserted upholstery spline onto the inserted foam slice cut spline, thus anchoring the upholstery to the foam and the frame.

- Start at top and bottom centers working outwards from center to each side continuing around the foam/frame corners, first gently stretching the upholstery over the foam to gain the desired contour and hog ringing the upholstery onto the frame bottom.

- Cautiously alternate hog ringing locations gently pulling upholstery to evenly contour foam and eliminate unnecessary folds in the fabric.

- It will be necessary to occasionally use diagonal pliers (dikes) to cut hog rings off and relocate the fastening location of them in order to gain the correct stretching distance and foam contour.

- Using an extra layer of batting material to increase the foams bulk should be used for any areas of the upholstery/foam desired to have greater bulk, contour or tension.

PRECAUTIONARY WARNING:

Upon installation of your upholstery fabric over your new foam seat cushion onto the seat frames; please do consider the application of using heat. Upholstery must be stretched and hog-ring fastened in temps in excess of 80 degrees Fahrenheit. A hot summer's day or heat lamps are required for correct stretching and installation of the upholstery vinyl fabric over the seat foams and hog-ringing onto the seat frames. You have no other alternative, other than to be in a very warm temp environment or to use auxiliary heat when you hog-ring the upholstery over the foams onto the frames, otherwise you may tear the upholstery or seams. Please contact me directly with any questions regarding the installation.

Best Birding,

Jed Zimmerman

How Marine Service 
Cell# 612-708-2144
askhow@aol.com